SOFA Switching Fatality and Severe Injury Update – 2015 First Quarter

PLEASE POST IMMEDIATELY

- No fatalities in 2015 through March 09…SOFA’s mission is Zero Switching Fatalities

- Last switching fatality on October 8, 2014…152 days switching-fatality free through March 09, 2015

October 8, 2014 – BNSF – Colorado Springs, CO: A BNSF conductor was pinned between equipment while switching on industrial track. She was 42-years old with 3 years of service. [based on preliminary information]

- Lower switching fatality counts since 2011

197 Fatalities, by year: 1992 through 2014, full year; 2015, part year through March 09

- Two switching hazards to review: Close/No Clearance and Going between Rolling Equipment

Since 2010, 11 of the 18 switching fatalities (61 percent) have involved these two hazards, as partially based on preliminary information. See examples on next two pages, and remedies on the page thereafter

Switching Operations Fatality Analysis (SOFA)

- A voluntary, non-regulatory, railroad-safety partnership comprised of representatives from AAR, ASLRRA, BLET, FRA, and UTU

- Seeks to prevent switching Fatalities through education based on facts about causes

- SOFA is not part of a rulemaking or regulatory process

- Recognizes that all have responsibility for switching safety: employees, managers, and regulators

- SOFA’s goal is Zero Switching Fatalities achieved through education and non-punitive interactions among stakeholders

- Find SOFA reports and information at: http://www.fra.dot.gov/SOFA [accessed March 09, 2015]

Recent Cases Involving Close/No Clearance

October 8, 2014 – Colorado Springs, CO

[This case is based on preliminary information and is subject to change upon SOFA review. All other cases in this display are based on SOFA review]

A BNSF conductor was pinned between equipment while switching on industrial track. She was 42-years old with 3 years of service.

May 28, 2012 – Kenmare, ND

The conductor was riding the leading end of a shoving move and struck rail equipment left out to foul on an adjacent track, fatally injuring the conductor.

January 30, 2012 – Gary, IN

The foreman was riding the leading end of a shoving move and struck rail equipment on an adjacent track, crushing the foreman.

February 08, 2011 – Kankakee, IL

A conventional yard switching crew (Engineer, Conductor and Student Conductor-In-Training) was using the lead during a flat switching operation and kicking cars from west to east. Lite engines, with conventional crew, asked to come up the lead from east to west and after permission was given and switching was stopped conductor who was using the lead told the lite engine crew to be careful about a car that was close to fouling the lead. The lite engine got by, restored the switch to track 19 for track 19 and left the 18 track switch lined for the lead – the fouling car was on track 18. During the subsequent shove move east on the lead, the lead car diverged into track 19. The conductor was crushed between the car he was riding and the car on track 18.

September 02, 2010 – Bridgeport, NJ

A 53-year old conductor was caught and crushed between the rail car he was riding and a standing rail car that was left fouling his movement on an adjacent track.

May 31, 2010 – Kearny, NJ

Employee cut out the trucks on locomotive NJTR 4206 and failed to ensure any brakes were applied to the locomotive.

Recent Cases Involving Going between Rolling Equipment

July 31, 2012 – Mason City, IA

After kicking cars into a track, a switchman was crushed between the equipment he was working on and free-rolling equipment previously kicked into the track.

September 08, 2011 – Botkins, OH

When a 16,800 ton southbound iron ore train on a 0.5 percent grade, a following train crew was instructed to tie down its train and help the stalled train over the summit. After uncoupling the EOTD and attaching the single helper engine, the helper conductor released his “Three Point Protection” and the stalled train was moved 4700 feet, stopping with 40 of the 125 cars over the crest. After communicating with the head-end crew, the helper conductor detached his locomotive – instructing the helper engineer to back away from the train “half a car”. The helper engineer backed up and stopped about 30 feet from the train. The helper conductor was, again, granted “Three Point Protection.” He then safely coupled the air-hose of the EOTD and released his protection. As he was walking back to the helper engine, the rear portion of the standing train started to roll northward toward the standing helper engine. Perhaps unaware of the slack running out, toward his locomotive, the helper conductor may have placed himself in the foul, crossing the track, and was coupled between the rear car of the stalled train and the front knuckle of the locomotive.

August 15, 2011 – Kansas City, KS

A pitch catch RCL operation included a switchman who was in control of the movement, coupled between two rail cars in a hump yard bowl track while he was adjusting draw bars between two cars.

July 25, 2011 – Bedford Park, IL

A remote control operation switch crew (RCO) was coupling track 16 in Bedford Park, IL, from the west end. The track had a declining grade, from east to the west. The conductor has on the north side of the track and controlling the movement. The switchman was on the west end of the locomotive protecting the point. The conductor made three couplings and was 17 car lengths in the body of the track when a radio signal of “Man Down” was transmitted over the radio. The conductor was found lying on his back over the north rail, with his legs in the gauge of the track and between the 17th and 18th cars ahead of the locomotive with the couplers crossed with a fatal injury of his left lower abdomen.

July 13, 2010 – East Deerfield, MA

A 34-year old Pan Am yard conductor was crushed between the couplers of two cars while aligning miss-aligned drawbars and/or opening couplers during a switching operations.

Close/No Clearance and Going between Rolling Equipment

6 of the last 18 fatalities involved Close/No Clearance (a SOFA Advisory) – section 3.5.6 of 2011 SOFA Report

Find at: http://www.fra.dot.gov/SOFA [accessed March 09, 2015]

- Close and no clearances involve insufficient space:

- No Clearance: Insufficient space to avoid being struck if passing or being passed by an object, structure, or equipment.

- Close Clearance: Insufficient space to take evasive action to avoid being struck by moving equipment that derails into an object, structure, or other equipment.

- Close and no clearances can be permanent or temporary:

- Permanent Close/No Clearance: A fixed structure that remains in the same location from day to day, such as a building, loading dock, fence, post, beam, or other permanent structure, that an employee passes.

- Temporary Close/No Clearance: A movable object, including equipment on or near one track fouling another track, rolling stock on an adjacent track, stacks of cross ties, construction materials, and doors or gates left open, that passes by an employee or employee passes.

- Remedies include:

- Eliminate when possible. Safety engineering (as elimination) is the favorable approach. Other approaches, such as behavioral, should be thought of as intermediate safety steps.

- Sign with standardize signage, at an appropriate distance (not too close or far) and on the same side, with instructions on how to act.

- Improve lighting.

- Indentify through maps, job briefings, transference of knowledge from experienced to inexperienced employees, inspection before action is taken, reporting with follow up, and reporting of close calls.

- When operating look for close/no clearances, ride away from these hazards or dismount as appropriate, plan for possibility of a derailment with an escape strategy, and avoid distractions (unnecessary conversation, doing paperwork, or cellphone use).

5 of the last 18 fatalities involved Going between Rolling Equipment (a SOFA Recommendation/Lifesaver, and  also the subject of Federal Railroad Administration’s Safety Advisory 2011-02 and 2013-03)

Recommendation 1

Any crew member intending to foul track or equipment must notify the locomotive engineer before such action can take place. The locomotive engineer must then apply locomotive or train brakes, have the reverser centered, and then confirm this action with the individual on the ground. Additionally, any crew member that intends to adjust knuckles/drawbars, or apply or remove EOT device, must insure that the cut of cars to be coupled into is separated by no less than 50 feet. Also, the person on the ground must physically inspect the cut of cars not attached to the locomotive to insure that they are completely stopped and, if necessary, a sufficient number of hand brakes must be applied to insure the cut of cars will not move.

Discussion 1

This recommendation emphasizes the importance of securing the equipment. A thorough understanding by all crew members that the area between cars is a hazardous location, whether equipment is moving or standing, is imperative.

DATA SECTION – 2015 First Quarter Update

18 Recent Switching Fatalities, January 01, 2010 through March 09, 2015

- These 18 fatality cases occurred since the 179 cases (January 1992 through December 2009) which formed the basis of the 2011 SOFA Report

- The purpose in displaying is to identify any potential emerging issues concerning risk in switching operations. Please note the two cases marked ‘preliminary’ have not yet been reviewed by SOFA. Thus, event reasons may change upon review

| Year | Count | Date | City | State | Reviewed or Preliminary | Fatality Reasons: brief description Risks other than those listed are often involved. Cases marked ‘preliminary’ are subject to revision of event reasons. |
| --- | --- | --- | --- | --- | --- | --- |
| 2010 | 1 | 04/23/10 | Riverdale | IL | reviewed | Lack or Inadequate Job Safety Briefing |
|  | 2 | 05/31/10 | Kearny | NJ | reviewed | Close/ No Clearance |
|  | 3 | 06/10/10 | Doswell | VA | reviewed | Struck by Mainline Train; and Drugs and Alcohol |
|  | 4 | 07/01/10 | Meridian | MS | reviewed | Employee Tripping, Slipping, or Falling |
|  | 5 | 07/13/10 | East Deerfield | MA | reviewed | Going between Rolling Equipment |
|  | 6 | 09/02/10 | Bridgeport | NJ | reviewed | Close/ No Clearance |
|  | 7 | 09/04/10 | Mobile | AL | reviewed | Industrial Hazard; and Miscellaneous Causes |
|  | 8 | 10/11/10 | Orange | TX | reviewed | Inexperience; and Employee Tripping Slipping, or Falling |
| 2011 | 9 | 02/08/11 | Kankakee | IL | reviewed | Close/ No Clearance (cars left afoul) |
|  | 10 | 07/25/11 | Bedford Park | IL | reviewed | Going between Rolling Equipment; and Unsecured Cars |
|  | 11 | 08/15/11 | Kansas City | KS | reviewed | Going between Rolling Equipment; and Miscellaneous Causes |
|  | 12 | 09/08/11 | Botkins | OH | reviewed | Going between Rolling Equipment; and Unexpected Movement of Railcars |
| 2012 | 13 | 01/30/12 | Gary | IN | reviewed | Close/ No Clearance (cars left afoul); and Environment; and Industrial Hazard |
|  | 14 | 05/28/12 | Kenmare | ND | reviewed | Close/ No Clearance (cars left afoul); and Inexperience; and Failure to Confirm Route of Movement |
|  | 15 | 07/31/12 | Mason City | IA | reviewed | Going between Rolling Equipment; and Lack or Inadequate Job Safety Briefing; and Unexpected Movement of Railcars; and Unsecured Cars |
| 2013 | 16 | 02/16/13 | Cleveland | OH | reviewed | Inexperience; and Drugs and Alcohol; and Employee Tripping, Slipping, or Falling |
| 2014 | 17 | 06/24/14 | Birmingham | AL | preliminary | Derailment |
|  | 18 | 10/08/14 | Colorado Springs | CO | preliminary | Close/ No Clearance (cars left afoul) |
| 2015 | -- | -- | -- | -- | -- | No switching fatalities through March 09 |

Reasons for Switching Fatalities…reasons have remedies

SOFA has reviewed 195 switching fatalities (1992 through 2013). Currently, two more cases remain to be reviewed. Each case is assigned one or more general reasons pertaining to its occurrence. Altogether, the 195 cases have 348 reasons assigned. There are 19 general reasons for switching fatalities identified by SOFA, plus a miscellaneous category. More specific reasons (not shown below) are also assigned to cases as Possible Contributing Factors (PCFs). Reasons have remedies…switching fatalities need not occur!

| Twenty | SOFA Reasons/Type | Number | Percent |
| --- | --- | --- | --- |
| Reasons |  | of | of |
|  |  | Cases | All |
|  |  |  | Reasons |
| 1 | Inexperience (1.5 or less years of service) | 35 | 10.1% |
| 2 | Close/No Clearance (both temporary and permanent) | 34 | 9.8% |
| 3 | Industrial Hazard | 28 | 8.0% |
| 4 | Going between Rolling Equipment (as to adjust knuckles, drawbars, etc.) | 26 | 7.5% |
| 5 | Lack of, or inadequate, job safety briefing (particularly when the nature of work changes) | 25 | 7.2% |
| 6 | Employee Tripping, Slipping, or Falling (unusually from equipment) | 21 | 6.0% |
| 7 | Struck by Mainline Trains (when working along mainlines or doing a roll by inspection) | 21 | 6.0% |
| 8 | Communicate Before Action is Taken | 20 | 5.7% |
| 9 | Derailment | 17 | 4.9% |
| 10 | Unsecured Cars | 17 | 4.9% |
| 11 | Miscellaneous Causes | 16 | 4.6% |
| 12 | Free-Rolling Railcars | 14 | 4.0% |
| 13 | Unexpected Movement of Railcars | 14 | 4.0% |
| 14 | Protect Employees Against Moving Equipment | 13 | 3.7% |
| 15 | Equipment Defect | 11 | 3.2% |
| 16 | Struck by Motor Vehicle (often at a crossing) | 9 | 2.6% |
| 17 | Environmental Conditions | 8 | 2.3% |
| 18 | Failure to Confirm Route of Movement | 8 | 2.3% |
| 19 | Drugs and Alcohol | 7 | 2.0% |
| 20 | Electronic Devices | 4 | 1.1% |
|  | total | 348 | 100% |

SOFA-defined Severe Injury Update

Definition: Based on its interests (i.e., potentially involving the same factors as fatalities), Severe Injuries are defined by the SOFA Working Group as (1) potentially life threatening; (2) having a high likelihood of permanent loss of function, permanent occupational limitation, or other permanent disability; (3) likely to result in significant work restrictions; and (4) resulting from a high-energy impact to the human body. ‘Severe Injuries’ include amputation, dislocation of the neck, loss of eye, electric shock or burn, and fracture to any bone except the lower arm, fingers, foot, and  toes. 1997 is the first year these Injuries to train and engine service employees can be determined as defined by the interest of the SOFA Working Group. For more information, see Severe Injuries to Train and Engine Service Employees: Data Description and Injury Characteristics. July 2001.

Note: The definition of SOFA-defined Severe Injuries is not to suggest that other injuries and illnesses resulting from operations are not also ‘severe’ and/or cause hardship to employees.

1,845 SOFA-defined Severe Injuries, by year: January 1997 through December 2014

1,845 SOFA-defined Severe Injuries, by month, January 1997 through December 2014

245 Amputations (counts are included in Severe Injuries), by year: January 1997 through December 2014

1,845 SOFA-defined Severe Injuries, by month and year,

January 1997 through December 2014

Among SOFA Updates, counts previously presented may change based on revisions to FRA data. The latest month available from the FRA lags the calendar month of this Update by three months. Publically available FRA data used in this table were accessed on February 28, 2015

|  | 1997 |  | 1998 |  | 1999 |  | 2000 |  | 2001 |  | 2002 |  | 2003 |  | 2004 |  | 2005 |  | 2006 |  | 2007 |  | 2008 |  | 2009 |  | 2010 |  | 2011 |  | 2012 |  | 2013 |  |  | 2014 |  |  | totals |  | average |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JAN | 11 |  | 13 |  | 16 |  | 15 |  | 21 |  | 12 |  | 11 |  | 11 |  | 20 |  | 10 |  | 14 |  | 13 |  | 6 |  | 6 |  | 8 |  | 9 |  | 8 |  |  | 6 |  |  | 210 |  | 11.7 |  |
| FEB | 17 |  | 15 |  | 9 |  | 9 |  | 9 |  | 13 |  | 17 |  | 14 |  | 10 |  | 6 |  | 15 |  | 12 |  | 4 |  | 7 |  | 9 |  | 2 |  | 5 |  |  | 10 |  |  | 183 |  | 10.2 |  |
| MAR | 14 |  | 12 |  | 17 |  | 11 |  | 10 |  | 10 |  | 13 |  | 10 |  | 9 |  | 9 |  | 11 |  | 5 |  | 5 |  | 4 |  | 5 |  | 6 |  | 3 |  |  | 5 |  |  | 159 |  | 8.8 |  |
| APR | 8 |  | 10 |  | 6 |  | 10 |  | 12 |  | 6 |  | 9 |  | 13 |  | 10 |  | 7 |  | 8 |  | 9 |  | 5 |  | 7 |  | 5 |  | 2 |  | 4 |  |  | 6 |  |  | 137 |  | 7.6 |  |
| MAY | 6 |  | 12 |  | 8 |  | 8 |  | 12 |  | 14 |  | 9 |  | 6 |  | 6 |  | 8 |  | 3 |  | 7 |  | 1 |  | 7 |  | 8 |  | 4 |  | 5 |  |  | 7 |  |  | 131 |  | 7.3 |  |
| JUN | 9 |  | 10 |  | 8 |  | 11 |  | 8 |  | 5 |  | 10 |  | 9 |  | 7 |  | 11 |  | 5 |  | 3 |  | 6 |  | 4 |  | 2 |  | 6 |  | 2 |  |  | 6 |  |  | 122 |  | 6.8 |  |
| JUL | 9 |  | 14 |  | 10 |  | 8 |  | 10 |  | 7 |  | 6 |  | 10 |  | 5 |  | 12 |  | 8 |  | 1 |  | 4 |  | 4 |  | 5 |  | 3 |  | 7 |  |  | 5 |  |  | 128 |  | 7.1 |  |
| AUG | 13 |  | 10 |  | 11 |  | 14 |  | 8 |  | 10 |  | 7 |  | 14 |  | 10 |  | 10 |  | 13 |  | 5 |  | 4 |  | 5 |  | 5 |  | 1 |  | 5 |  |  | 7 |  |  | 152 |  | 8.4 |  |
| SEP | 10 |  | 11 |  | 15 |  | 10 |  | 20 |  | 12 |  | 5 |  | 4 |  | 9 |  | 6 |  | 10 |  | 12 |  | 5 |  | 3 |  | 4 |  | 5 |  | 4 |  |  | 3 |  |  | 148 |  | 8.2 |  |
| OCT | 12 |  | 12 |  | 16 |  | 10 |  | 5 |  | 11 |  | 9 |  | 7 |  | 11 |  | 5 |  | 11 |  | 4 |  | 2 |  | 4 |  | 4 |  | 1 |  | 6 |  |  | 9 |  |  | 139 |  | 7.7 |  |
| NOV | 12 |  | 9 |  | 12 |  | 11 |  | 13 |  | 14 |  | 10 |  | 10 |  | 13 |  | 8 |  | 6 |  | 8 |  | 3 |  | 6 |  | 9 |  | 3 |  | 5 |  |  | 7 |  |  | 159 |  | 8.8 |  |
| DEC | 18 |  | 9 |  | 7 |  | 22 |  | 12 |  | 9 |  | 8 |  | 15 |  | 12 |  | 8 |  | 6 |  | 8 |  | 8 |  | 6 |  | 5 |  | 5 |  | 14 |  |  | 5 |  |  | 177 |  | 9.8 |  |
| totals | 139 |  | 137 |  | 135 |  | 139 |  | 140 |  | 123 |  | 114 |  | 123 |  | 122 |  | 100 |  | 110 |  | 87 |  | 53 |  | 63 |  | 69 |  | 47 |  | 68 |  |  | 76 |  |  | 1,845 |  | 102.5 |  |

245 Amputations (a type of Severe Injury), by month and year,

January 1997 through December 2014

A type of SOFA-defined Severe Injury, Amputations are displayed separately because of the extreme trauma to employees engaged in switching, and the likelihood of permanent occupational and lifestyle limitations. Counts for Amputations are contained in the counts of SOFA-defined Severe Injuries (shown on previous page). Publically available FRA data used in this table were accessed on February 28, 2015

|  | 1997 |  | 1998 |  | 1999 |  | 2000 |  | 2001 |  | 2002 |  | 2003 |  | 2004 |  | 2005 |  | 2006 |  | 2007 |  | 2008 |  | 2009 |  | 2010 |  | 2011 |  | 2012 |  | 2013 |  |  | 2014 |  |  |  | totals |  | average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JAN | 1 |  | 0 |  | 2 |  | 1 |  | 0 |  | 0 |  | 2 |  | 2 |  | 2 |  | 0 |  | 1 |  | 1 |  | 1 |  | 0 |  | 2 |  | 0 |  | 0 |  |  | 0 |  |  |  | 15 |  | 0.8 |
| FEB | 0 |  | 1 |  | 0 |  | 1 |  | 0 |  | 2 |  | 1 |  | 2 |  | 0 |  | 2 |  | 1 |  | 0 |  | 0 |  | 1 |  | 2 |  | 0 |  | 1 |  |  | 1 |  |  |  | 15 |  | 0.8 |
| MAR | 3 |  | 4 |  | 3 |  | 2 |  | 1 |  | 1 |  | 3 |  | 1 |  | 2 |  | 1 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  | 0 |  |  | 1 |  |  |  | 25 |  | 1.4 |
| APR | 1 |  | 2 |  | 0 |  | 1 |  | 2 |  | 0 |  | 1 |  | 1 |  | 2 |  | 2 |  | 3 |  | 3 |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  |  | 0 |  |  |  | 20 |  | 1.1 |
| MAY | 1 |  | 2 |  | 3 |  | 0 |  | 2 |  | 2 |  | 2 |  | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  | 2 |  | 0 |  | 2 |  |  | 2 |  |  |  | 21 |  | 1.2 |
| JUN | 2 |  | 1 |  | 1 |  | 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  | 1 |  |  |  | 10 |  | 0.6 |
| JUL | 1 |  | 5 |  | 1 |  | 0 |  | 4 |  | 0 |  | 1 |  | 2 |  | 1 |  | 2 |  | 2 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  |  | 2 |  |  |  | 24 |  | 1.3 |
| AUG | 1 |  | 0 |  | 1 |  | 4 |  | 0 |  | 1 |  | 0 |  | 2 |  | 2 |  | 0 |  | 3 |  | 0 |  | 1 |  | 1 |  | 0 |  | 0 |  | 1 |  |  | 1 |  |  |  | 18 |  | 1.0 |
| SEP | 2 |  | 4 |  | 3 |  | 2 |  | 5 |  | 4 |  | 0 |  | 0 |  | 3 |  | 1 |  | 1 |  | 2 |  | 0 |  | 1 |  | 0 |  | 2 |  | 0 |  |  | 1 |  |  |  | 31 |  | 1.7 |
| OCT | 2 |  | 5 |  | 2 |  | 2 |  | 0 |  | 0 |  | 2 |  | 2 |  | 0 |  | 0 |  | 2 |  | 0 |  | 0 |  | 1 |  | 1 |  | 1 |  | 2 |  |  | 2 |  |  |  | 24 |  | 1.3 |
| NOV | 2 |  | 2 |  | 2 |  | 2 |  | 3 |  | 0 |  | 1 |  | 1 |  | 2 |  | 3 |  | 1 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  |  | 2 |  |  |  | 22 |  | 1.2 |
| DEC | 4 |  | 1 |  | 0 |  | 4 |  | 1 |  | 1 |  | 2 |  | 1 |  | 1 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 1 |  | 2 |  | 1 |  |  | 0 |  |  |  | 20 |  | 1.1 |
| totals | 20 |  | 27 |  | 18 |  | 19 |  | 19 |  | 11 |  | 15 |  | 15 |  | 15 |  | 12 |  | 16 |  | 8 |  | 6 |  | 6 |  | 11 |  | 6 |  | 8 |  |  | 13 |  |  |  | 245 |  | 13.6 |

Switching Fatalities, SOFA-defined Severe Injuries, and Other Reportable Events

Source: Switching fatalities from SOFA Database; all other series from FRA, accessed February 28, 2015

Note: Among SOFA Updates, counts previously presented may change based on revisions to FRA data

| Year | SOFA Switching Fatalities | SOFA-defined Severe Injuries | Amputations (counts are included in SOFA-defined Severe Injuries) | All Employee On-duty Fatalities less SOFA Switching Fatalities | T&E Employee On-duty Fatalities less SOFA Switching Fatalities | All Reportable Employee Casualty to T&E Employees (includes Fatalities and Severe Injuries) | All Accidents | Human Factor Accidents | Highway-Rail Crossing Incidents | Trespasser Incidents (not at crossings) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1992 | 14 | * | * | 20 | 6 | 6,648 | 2,359 | 864 | 4,910 | 1,049 |
| 1993 | 15 | * | * | 32 | 16 | 5,649 | 2,611 | 865 | 4,892 | 1,032 |
| 1994 | 12 | * | * | 19 | 9 | 5,026 | 2,504 | 911 | 4,979 | 981 |
| 1995 | 11 | * | * | 23 | 10 | 4,215 | 2,459 | 944 | 4,633 | 955 |
| 1996 | 7 | * | * | 26 | 15 | 3,726 | 2,443 | 783 | 4,257 | 945 |
| 1997 | 11 | 139 | 20 | 26 | 10 | 3,489 | 2,397 | 855 | 3,865 | **1,049 |
| 1998 | 8 | 137 | 27 | 19 | 8 | 3,642 | 2,575 | 971 | 3,508 | **1,049 |
| 1999 | 9 | 135 | 18 | 22 | 12 | 3,835 | 2,768 | 1,031 | 3,489 | 924 |
| 2000 | 13 | 139 | 19 | 11 | 2 | 3,893 | 2,983 | 1,147 | 3,502 | 877 |
| 2001 | 8 | 140 | 19 | 14 | 6 | 3,561 | 3,023 | 1,035 | 3,237 | 915 |
| 2002 | 6 | 123 | 11 | 14 | 3 | 3,022 | 2,738 | 1,050 | 3,077 | 935 |
| 2003 | 10 | 114 | 15 | 9 | 3 | 2,935 | 3,019 | 1,230 | 2,977 | 896 |
| 2004 | 11 | 123 | 15 | 14 | 9 | 2,910 | 3,385 | 1,353 | 3,085 | **878 |
| 2005 | 11 | 122 | 15 | 14 | 7 | 2,817 | 3,266 | 1,270 | 3,066 | **878 |
| 2006 | 7 | 100 | 12 | 9 | 0 | 2,483 | 2,998 | 1,068 | 2,942 | 992 |
| 2007 | 6 | 110 | 16 | 11 | 4 | 2,520 | 2,693 | 1,047 | 2,778 | 877 |
| 2008 | 12 | 87 | 8 | 14 | 4 | 2,217 | 2,481 | 910 | 2,429 | 889 |
| 2009 | 8 | 53 | 6 | 8 | 2 | 1,972 | 1,912 | 656 | 1,933 | 760 |
| 2010 | 8 | 63 | 6 | 12 | 5 | 1,882 | 1,902 | 650 | 2,052 | 830 |
| 2011 | 4 | 69 | 11 | 17 | 11 | 1,735 | 2,022 | 746 | 2,061 | 772 |
| 2012 | 3 | 47 | 6 | 13 | 4 | 1,547 | 1,760 | 661 | 1,985 | 824 |
| 2013 | 1 | 68 | 8 | 13 | 2 | 1,760 | 1,820 | 689 | 2,096 | 863 |
| 2014 | 2 | 76 | 13 | 8 | 2 | 1,872 | 1,713 | 663 | 2,280 | 945 |
| % change | -- | -- | -- | -- | -- | 6.4% | -5.9% | -3.8% | 8.8% | 9.5% |

*SOFA-defined Severe Injuries are defined only back to 1997		**Counts happened to be identical for these successive years